VOIP lab experiment#6

Creating a trunk in PBX

Installation and Repair of Telecommunications Equipment

Name of student:________________

Group Number:_________________

Date:_________________________

The purpose of the lab is to create a trunk in the PBX. A trunk is the phone number of the supplier such as Bell, Videotron, etc…

Go to the trunk, inside of the menu connectivity, and then click on add a sip trunk

- Enter the following information:

OUTGOING********

host=172.16.0.2

username=51455500xx

fromuser=51455500xx

secret=pec123

type=peer

context=from-trunk

insecure=very

**************

Incoming settings******************

USER CONTEXT 51455500xx

type=peer

secret=pec123

host=172.16.0.2

allow=all

insecure=very

*************************************

Register String:

51455500xx:pec123@172.16.0.2/51455500xx

click on submit and then apply configuration.